1781 Contract to build a 74 gun ship (Launched 1786)

NOTE: Three sets of numbers were shown for many items, the top and bottom appearing to have been added after the original contract was prepared. Speculation is that emendations were written above the number in the contract.  If this is the case, they may have come from the Master Shipwright. As-built dimensions would have been written below the original number in the contract.   Contract dimensions and as-built dimensions (in parentheses) are used when available although they were sometimes extremely difficult to read.  This is transcribed from a hand written contract.  Lacking a contract for other ships of this class, the original dimensions in this contract may be of help for modeling ships within the Arrogant class.  When transcribing this hand written contract it was interesting to find the handwriting change every few pages as there were obviously several persons doing the writing.  Use of a capital letters for nouns varied, the use of abbreviations, and spelling sometimes varied although this might have just been as simple as poor spelling on the part of one or two of the writers.

LENGTHS…….  	LENGTH of the keel for tonnage 139 feet 1 inch (138 feet 3⅜ inch) and 				shall not be accounted more for casting the Ship’s Tonnage.						On the Gun Deck from the aftside of the stem, to the Rabbit of the Stern 				Post 168 feet 0 inches (168 feet 6 inches.)

BREADTHS….	Extreme, from Out to Outside of the Plank of the Bottom  in Midships 47 feet 3 inches (47 feet 1 inch,) or to be moulded in midships 46 feet 4 inches (46 feet 2 inches) inches and shall not be accounted more for casting the Ship’s Tonnage

Breadth moulded at the Height of Breadth at the aftermost part of the Counter 27 feet 9 inches (31 feet 0 inches)

Breadth moulded at the Top Timber Line, or upper side of the Waste Rail 	at the beakhead bulkhead 27 feet 5 inches (27 feet 6 inches) In  Midships 36 feet 7 inches (36 Feet 8 inches) , and Abaft 21 feet 6 inches (21 feet 8 inches)

DEPTH in HOLD 	Taken from the upper side of the Strake next the limber boards, to the upper side of the Gun Deck Beams in midships18 feet 10 inches (19 feet 9 inches).

STRAKE next the	Thick 7½ inches (7½  inches), broad 1 foot 2½  inches

LIMBER BOARDS

BURTHEN in TONS  	1634 21/94  (1612 8/94)

DRAUGHT of 	Afore 20 feet 1 inch, (20 feet 7 inches) Abaft 20 feet 1inch (21 feet

WATER                          inches)

HEIGHTS	Of the cutting down in Midships above the upper side of the Main Keel

1 feet 7 inches (1 foot? inches)

Height of the Gun Deck by a Perpendicular from the upperside of the Keel to the upper side of the Plank, at the Middle Line of the Deck at the Aft Part of the Stem 23 feet 4 inches,(24 feet? inches)  at the Foreside of the Stern Post 23feet 4 inches. (25 feet 8 inches)

From the upper side of the Keel to the upper side of the Wing Transom at the Stern Post 25feet 8 inches (27 feet 4 inches).

From the upper side of the main Keel to the Angle of the Lower Counter at the Middle Line 30 feet 5 inches. (32 feet 1 inch)

Of Breadth above the upper side of the Main Keel in Midships Lower 17 feet 6 inches, (18 feet 8 inches) Upper 20 feet 5 inches (21 feet 8 inches)

Of the Top Timber Line or Upper side of the Waste Rail above the upper side of the main Keel at the Beakhead Bulkhead 36 feet 6 inches (37 feet 8 inches.  In Midships 34 feet 0 inches (35 feet 0 inches); at the Stern Timber 42feet 0 inches (43 feet 4 inches)

Height of the Rising in Midships above the upper side of the Keel 4 ½ inches (11 inches)

ORLOP………..	Height from the Upper Side of the Beam, to the Lower Side of the Gun Deck Plank at the Middle of the Beam  6 feet 6 inches (6 feet 10 inches)

Beams to round in Midships 2 inches (2 ½ inches)

PLATFORMS……	Height between the Upper Side of the Plank or Deal on the Platform, to the Lower Side of  the Gun Deck Plank at the Middle Line of the Deck, Fore Platform 6 feet 6 inches (6 feet 10 inches,) After Platform 6 feet 6 inches (6 feet 10 inches.)

GUN DECK….	Beams to round 4feet ¾ inches (5 inches) 	Plank Thick 4 inches.

Height from the Plank of the Gun Deck to the Upper Side of the Upper Deck Beam at the Middle of the Beams, Afore 7 feet 0  inches, (7 feet 0 inches) Midships Afore 7 feet 0  inches, (7 feet 0 inches), and Abaft Afore 7 feet 0  inches, (7 feet 0 inches.)

Height from the upperside of the Gun Deck Plank to the upperside of the low Port Cills 2 feet 4” (2 feet 4 inches)

Port deep 2 feet 8 inches (2 feet 8 inches), for & aft 3 feet 5 inches (3 feet 5 inches.) Ports for the water in midships 5 feet 3 inches (5 feet 6 inches)

UPPER DECK…..	Height from the Upper Side of the Plank of the Upper Deck, to the Upper Side of the Quarter Deck Beams, at the Middle of the Beam, Afore 6 feet 7 inches (6 feet 9 inches) Abaft 6 feet 9 inches 6 feet 11 inches). Height of the waste from the upperside of the Plank of the Deck at the lowest place 5 feet 10 inches (5 feet 11 inches). Height of the upper side of the lower port cill from the upperside of the plank of the upper Deck, 1 foot 11 inches (2 foot 11 inches)

Ports deep 2 feet 8 inches (2 feet 8 inches) fore & aft 2 feet 10 inches (2 feet 10 inches).

Beams to round 7½  inches (7½ inches) Plank thick 3 inches  Height form the upper side of the Plank of the upperdeck to the upperside of the Forecastle Beams at the middle of the Beam afore 6 feet 4 inches (6 feet 4 inches) abaft 6 feet 4 inches (6 feet 4 inches)

FORECASTLE	Beams to round 6½  inches (7 inches) Plank thick 3 inches (3 inches)

Long taken at the fore side of the Stantions of the Beakhead Bulkhead to the aftside of the after Beam at the middle line of the Deck 40 feet 6 inches.

QUARTER DECK…	Beams to round 8½ inches (9 inches) Plank thick 3 inches (3 inches)

Height from the Plank of the Quarter Deck, to the Upper Side of the Roundhouse Beams at the middle of the Beam; afore 6 feet 5 inches (6 feet five inches), abaft 6 feet 7½ inches (6 feet ? inches.)

Height from the Plank of the Quarter Deck, to the Upper Side of the lower Port Cill 1 feet 8 inches (1 feet 8 inches.)

Ports deep 2 feet 5 inches, (deep 2 feet 5 inches) Fore and Aft 2 feet 8 inches (2 feet 8 inches.)

.

The Quarter Deck to be continued in Length so far forward as that the Foremost Beam may receive a Tenon in the underside from the Head of the Main Topsail Sheet Bitts, and if occasion to return Aft at the side as much as may be necessary for the Ladder to the entering place standing clear of the gun and that the second or third Beam also receive a tenon from the Head of the Jeer Bitts.

ROUND HOUSE	Beams to round 11 inches (11 inches).  Plank of the Deck, thick 2½ inches (2½ inches). Length taken in midships from the aftside of the Stern Timber to the foreside of the foremost beams 48 feet 0 inches

SHIP’s EXTREME	  Length from the aftside of the Taffrail, at the Height of the Fiferail, to the LENGTH …….	  Forepart of the figure of the Head by a Line Parallel to the Keel 197 feet 10 			  inches (198 feet 4 inches.)

Length from the Foreside of the Stem to the Foreside of the Knee of the Head 14feet (13 feet 11 inches.)

The Beakhead to the foreside of the beakhead Bulkhead 6 feet 8 inches. (6 feet 7 inches.)

Length from the Aftside of the Wing Transom to the Aftside of the Stern Timber at the Angle of the Lower Counter at the Middle Line 6 feet 6” inches (6 feet 1 inch)

Length from the Aftside of the Wing Transom to the Aftside of the Stern Timber at the Angle of the Second Counter at the Middle Line 8 feet 7 inches (8 feet 0 inches.)

ROUNDING of the	  Rounding of the Stern Aft at the Upper Side of the Wing Transom 10½ 	STERN	  inches.  Rounding of the Stern aft at the upper counter rail (?  Contract is 				  torn and dimension is missing )

KEEL …………	The Keel to be Elm, not more than 6 pieces if to be got, otherwise to have a short piece abaft of it about 22 feet, if required to be of oak, the foremost piece about 24 feet and the intermediate pieces to be as near of a length as can be made suitable to the whole of the what remains. To be 1 feet 6 inches square in the midships, sided afore 1 feet 2 inches, and at the Aft Part of the Rabbet of the Post 12½ inches. The Scarphs 4 feet 6 inches (4 feet 6 inches )long, tabled one into the other, laid with white Flannel or Kersey, and bolted with eight Bolts 1¼ inches in diameter.  The Lips of the said Scarphs not to be left more n thickness than 5¼ inches.

FALSE KEEL…	The False Keel to be of one Thickness 5 inches (7 inches) thick, so as  to make the main and false Keels together 1 feet 7 inches (1 foot 10 inches) below the Rabbit, to give Scarph to the Scarphs of the Main Keel to be laid with Tar and Hair, and sufficiently fastened with Nails and Staples.  The Sides and Bottom to be filled or sheathed with Copper and to have thin Copper put between the Main and False Keels all Fore and Aft properly tuned up and fastened, and sufficiently fastened with Nails & Staples.

STEM	Not to take more than 3 pieces of good sound Oak, quite free from Defects of any kind, to be thwartships at the head 2 feet (2 feet 2 inches) and to diminish from thence to 1 feet 6 inches at the lower side of the lower cheek and at the end of the Keel or reconciling of the sweep. 1 foot 2 inches to be molded at the head 1 foot 6 inches and at the fore foot the same as the Keel, the scarphs 4 feet long, abled one into the other, laid with white flannel and Kersey and bolted with 6 bolts of 1⅛ inch diameter.  Two of the middle bolts to go through the false Stem and well clenched thereon, the Lips of the Scarphs not to be more than 5¼ inches thick.  The rabbit to be taken out agreeable to what is expressed in the draft.

FALSE STEM …..	The false Stem or Apron to be 11 inches in thickness  and the  breadth of the Stem as far down as the Knightheads fay to it, and below as the body may require the scarphs to be 12 inches long.

STERN POST	The Stern Post to be of good sound Oak Timbers of the best kind, free from Defects, square at the Head 1 feet 11 inches, (1 feet 11 inches) Fore and Aft on the Keel, the back or false Post, if any, included 2 feet 10 inches (2 feet 8½ inches) abaft the Rabbit at the Wing Transom 11½ inches (11 inches), and on the Keel 1 feet 2 inches, not to have less than 11 inches Main Post abaft the Rabbit, the Breadth athwartships at the Head to be continued down as low as the deck Transom, & from thence to begin its tapering to 1 foot ½ inches, as before observed, that the main Keel  to be wrought at that Place, not to be bearded until the Back is on, & then to be no more below than the tapering of the keel requires, and instead of a Dovetail  Plate as usual for the security of the lower part of the stern post with the keel that there be wrought a square Plate of Iron of the knee kind the flat way of 4 ½ inches in breadth & ⅝ inches in thickness the up and down Arm to be 3 feet 6 inches long & bolted with 3 bolts of ⅞ inches diameter .  The fore and aft Arm to be 5 feet long, let in it thickness to the Keel and Post, and carefully bolted through, and clenched on the Plate

INNER POST ……	The Inner Post to run up to the Under Side of the Deck Transom to be 13 inches (13 inches) fore and aft there, and 1 feet 5 inches (1 feet 5 inches) on the Keel, to be of the same Breadth athwartships, as the main Post from the lower Transom up, from thence down as the Body requires.

WING TRANSOM …	The Wing transom to be sided 1 feet 1½ inches, moulded at the ends 1 feet 6 inches, in the Middle 2 feet, no Chocks to be admitted on the Aftside, and to be bolted to the Post with 2 bolts of 1¼ inch diameter, or (if the Standard Bolt comes in the Wing Transom) as shall be directed.

FILLING Ditto	the Transoms between the Wing and Deck to be sided 10 inches, and to be left for Air between the Wing, and filling 2½ inches, and between the       filling and Deck Plank 3inches, and to be bolted to the Post with 1 Bolt of

1¼ inches diameter.

DECK   Do	The Deck Transom to be sided1 foot 0½ inches and moulded as broad as may be better fastening the Plank of the Deck, to be bolted to the Post with 2 Bolts of 1¼ inches diameter. The chocks on the aftsides (if any) no to exceed in thickness 7 inches.

OTHER TRANSOMS	To have 5 whole Transoms below the deck Transom, free from Shakes and Defects, sided 12 inches. To lie 3½ inches clear of each other for air, the chocks on the Transom next the Deck transom not to exceed 12 inches, or that on the lower 1 foot 6 inches.  The Transoms to be bolted to the post with 1 bolt in each of 1 ½ inches diameter.      .

FASHION PIECES	The Fashion Pieces and Timbers afore and abaft to be canted so as to be near a Square as possible, and agreeable to what is expressed on the Draught: The Foremost to run up at least 2 feet 6 inches above the Wing Transom and down to the upper edge of the lower transom, and sided 13 inches, the After one to run up to the Underside of the Transom next below the Deck Transom, and the stepping on the Deadwood.

DEAD and RISING   	To have sufficient Number of pieces of Rising Wood on the Keel

WOOD	 in Midships 8 inches thick and 1 feet 10 inches broad, and of Dead Wood afore and abaft for Security of the Half Timbers, of a Height to answer the Run of the Keelson, to give proper Shift to the Scarphs of the Keel, and to each other the first or lower pieces of Deadwood on the Keel abaft to Tenon into the inner Post with two Tenons at the after end.

KNEE of the	To have a Knee of sufficient Substance against the Stern Post

DEADWOOD 	upon the Deadwood, the  up and down Arm of the knee to be 7 feet and fore and aft Arm 10 feet 6 inches and bolted at every 22 inches Distance with the Bolts of 1½ inches diameter through the Stern Post, and thereon to bring the Rest of the deadwood, and to fasten it with the same sized Bolts at the aforesaid Distance, or at the Moulding Edge of each Half Timber all through the Keel, and properly placed to secure the after End of the Keel, and lower End of the Stern Post, and that all that is possible may go through by one sized auger for their being on any Repair drove out

BOLLARD TIMBERS  To have proper Bollard Timbers for Security of the Bowspreet sided rom thence at the head of the stem  1 foot 4 inches, and at the lower ends which are to run down to take a bolt in the hook below the gun deck hook 13 inches and from the head of the stem up to be open to the diameter of the bowspreet  from thence down to fay to the stem. And on each side

HAWSE PIECES..	To have on each Side 4 Hawse Pieces, the Foremost and Aftermost sided 1 foot 5 inches (1 foot 6 inches) inches each, and 2 middle Pieces 1 foot 6 inches (1 foot 7 inches)each, the Diameter of the Hawse Holes to be 1 foot three inches inches after the Hawse Pipes are let out, and 1 foot 6 inches asunder on a square.  The side of each hawse piece to be wrought in the direction of the holes fay’d close to each other in wake of the holes, and quite free from sap or wain but the stuff to run thro’ within and without board, and the Hawse holes lined with lead of 1 inch thick at the lower part to answer the end of  boxing.

FRAME	It being of the utmost consequence to the strength of the Ship that all the Timbers of the Frame should as much as possible be preserved from being cut by the Ports of each Deck.  A Disposition for that purpose is made on the Draught, and as a further means to obtain it, the said Timbers appointed to make the sides of the Ports being continued up to the Top of the side are to be framed in bends, 1st, 2nd, 3rd, & 4th. Futtocks and Top timbers together (as distinguished on the Draught) and fastened with three Bolts of 1 ¼ inch in each scarph and that the first futtocks be bolted to the respective floors of every bend with three bolts of 1 ¾ diameter, that the stations of the said frames of them as stand afore and abaft so distinguished are to be preserved with great exactness and for such of them as stand afore and abaft the square frames, and are to be canted, these also are to keep their stations at the breadth, and what is required to give the fashion piece and Beakhead Timbers a proper flight or cant according to the turn of the Body, should be done by reducing the Room and Space on the side of the Keel.  And as by this disposition of the Frames in their several Stations they are respectively appointed to Stand, every Gun Deck Port, and one side of every Upper Deck Port is at once provide for by them as one 4th futtock and one Toptimber proper will make the side of every Gun Deck Port, one Toptimber proper and either a 4th Futtock continued up (when can be got of a length) will make the sides of every upper Deck Port, but if the 4th futtock cannot be got of a length to run up to perform that office, the said side of such upper Deck Port is to be made by the Toptimbers which come und the Port being formed in a cast over the 4th futtock head, and run up taking care that the said cast be so disposed that it be not cut so as to hut the Timber by the lower Cill of the Port, or as shall be directed.  And as all Ships should be kept as light as possible in her upper works, consistent with the Service required of them, so the Frame should not be encumbered with a Timber more than is necessary for the several offices for which they may be wanted, and if 2 Short Timbers (and by no means more) sided at the lower end 11 inches and at the upper end 9½ inches stands upon the cill of every Gun Deck Port, properly placed to receive and upper deck Standard Quarter Deck Port or such other service necessary to their Stations and there be also 2 short Timbers admitted to stand on the cill of every upper deck Port, sided 10 inches at the lower and 9 inches at the upper end, which with such as are necessary to make the Gallery Doors from the Quarters make the sides of the Quarter Deck Ports, filling in between the foremost Frame and Hawse pieces the whole ships Frame will be sufficiently full every purpose answered and not the least occasion for more every Frame Bend to be separated with pieces of dry Oak in wake of the Bolts for bolting the frames together.

ROOM and SPACE   	The Room and Space of the Timbers to be 2 feet 8 ⅞ inches, or agreeable to the Stations  of the Frames on the Draught which in distance from the perpendiculars cutting the aftside of the Stem, and fore part of the Rabbit of the Stern Post shall most exactly conform to what is therein expressed.

FLOOR TIMBERS	The Floor Timbers, between Timber 5 and Timber E in the Bearing of the Ship	sided 1foot 3½  inches, and from  5 to 15 aft, and from E to L forward to be sided 1 foot 2½ inches, and from thence aft and forward 1 foot 2 inches and in the wake of the half Timbers 1 foot or as the space of the Cant Timbers will admit, to be in length in Midships 26, and Afore and Abaft as the Draught directs.  No Chocks to be larger on any Floor Timber than will  admit of 1 foot 3½ inches whole Wood below the cutting down in Midships, and Afore and Abaft to increase as the Shape of the Timber may require for Strength, and to be moulded at the Heads 1 foot 1½ inches, and Afore and Abaft the same. Every Floor Timber to be bolted through the Main Keel. And the Bolts carefully clenched before the false Keel is put under with Bolts of 1½ diameter.

LOWER FUTTOCKS   	The Lower Futtocks in Midships from I and 11 forward to be sided 1 foot 3 inches, and from I to the  foremost floor forwards and from 11 to the aftermost floor aft to be sided 1 foot 2 inches and in the wake  of the half Timber according to the room and Space, the Cant Timbers allow that they, the said futtocks run down to the Deadwood, and not less than 6 inches whole Wood there, to have Chocks across the Heels of the them to supply the Wood that may be wanting from thence to the cutting down or Inside of the Floor Timbers.  And that they scarph on the second Futtock  in Midships   6 feet 11 inches and afore and abaft, 6 feet or agreeable to what is expressed on the Draught, and to be in and out and moulded at the Heads 12 ⅞ inches.

SECOND FUTTOCKS The Second Futtocks to be sided between Timbers 13 and I, 1 foot 2¼  inches, from thence forward and aft 1 foot 1¾ inches, or in the wake of Cant Timber, according to what is required from the room and Space of the them to have 6 feet 11 inches Scarph in the Midships to the 3rd  Futtock, and as the Draught directs Afore and Abaft; and to moulded at the Heads  12 ¼ inches, not to have less than 3 ½ inches whole wood at the Heels if double futtocks and half Timbers afore and abaft.

THIRD FUTTOCKS     The Third Futtocks to be sided  1 foot 1 ¾ inches to give Scarph to the Fourth Futtocks 6 feet 11 inches in Midships, and not less than 6 feet Afore and Abaft that those Timbers that come under the Gun Deck Port, run up to the cill and to be moulded at the run of the third futtock heads 11 ⅝ inches and in and out at the Gun Deck 11½ inches.

FOURTH FUTTOCKS The upper or 4th Futtocks in the Midships  to be sided 1foot 1½ inches, to have scarph to the Toptimber in Midships 11 feet 6 inches and not less than 9 feet 10 inches afore and abaft, and to be moulded or in and out at this range of the Upper Deck waterways 10 ½ inches that all those appointed to make the sides of the Gun Deck and Upper Deck Ports be well grown young strong Timber, free from Sap or Wain, and as the bigness of the Gun Deck Port is 3 feet 5 inches fore and Aft, and the room and space of the two frames on each side them 5 feet 5 ¾ inches those must remain full 12 inches timber from the moulding side of each 4th Futtock or Top timber to the side of the Port, after the Port is trimmed out to that bigness, and therefore great care to be observed that such were appointed to make the sides of Ports conform in the siding accordingly, that when the Port is formed to its bigness fore and aft not the least sap or wain be found in the Timber in Wake thereof, and that all such as come in the Wake of the main and Fore Channels, or that make the sides of gun or Upper Deck Ports run up to the Top of the side, or at least as many of them as can be got, and that none of those that make the sides of the Gun Deck Ports be less than 3 feet square Timber above the upper Cill of the Gun Deck Port.

TOP TIMBERS…	The Top Timber in Midships to run down to the lower edge of the upper strake of Gun Deck Clamps to be sided at the Heels and at the upper Futtock Heads 1 foot 1 ½ inches, and at the top of the side 1 foot, except such as are appointed to make the sides of the upper Deck Ports which are to be continued the same bigness as below, or according to the Room. There is for them and for those Top Timbers that are by long frames to make the sides of Gun Deck Ports the same care to be observed in respect to their bigness always between the Moulding of the Frame and the side of the Port as has been mentioned of the upper Futtocks and that those which are to make the sides of upper Deck Ports – instead of such 4th Futtocks as cannot run up to do that office and requires to have a cast over the fourth Futtock Heads to clear itself of the side of the Port for that purpose to take care that it be well grown to form the said cast and the cast disposed of proper to the height of the port, that the Timber may not be wounded thereby and that not the least sap or wain be left remaining in the Top  Timber, in the Wake of the Gun Deck and upper Deck Ports, when the ports are trimmed out, and the whole to be well grown, young strong Timber and Moulded at the Top of the side at the run of the Ribband at the Top Timber, breadth in Midship 5⅛ inches abaft 5⅝ inches and at the Beakhead 6 ¼ inches and at the Fife rail from the 2nd Drift to the stern Timber to be moulded 4 ½ inches and from the Drift forward in wake of the Timber Heads 5 ½ inches.

TIMBER OF   	That all the Timber of the Frames be well grown, young, sound and

THE FRAME	good free from Defects of any kind wrought square as much as possible without sap or wain and great exactness used in forming the same that it may conform to this Draught which accompanies the Contract, that those Timber between the Frames not appointed to Ports be justly disposed to divide the spaces as equal as possible and every Timber to stand on its proper head.  The Heads and Heels of all Timbers be square after wrought not less than one third of the moulding way.

KEELSON	The Keelson to be square 1 foot 6 inches, scored down on the Floor Timbers, not more than 6 pieces, to give proper shift to the Scarphs of the Main Keel, the scarphs to be in Length 5 feet 5 inches, and so disposed that the Middle of  each Scarph be over a Floor Timber that is not bolted that a keelson bolt may go through the middle of the scarph, that the Scarphs be Hook and Butt, and the Lips of the Scarphs in Thickness but 5 inches, and bolted with 2 bolts of  ⅞ inch diameter  through the Keelson & about 6 inches into the Floor Timbers. The other parts of the Keelson to be bolted thro’ every other Floor Timber thro’ the Main Keel with Bolts of 1 ½ inch diameter and carefully clenched there.

STEMSON …..	To have a Stemson or Inner Stem, square 13 inches to run up to the lower side of the upper Deck Breast hook and give shift to the Scarphs of the Stem and Apron below, and the lower end to Scarph to the foremost pieces of the Keelson

PLANK of the	The Plank of the Bottom to be 4 inches thick of English, the best

BOTTOM 	of its kind, from the light Draught of Water up, cut regular, and well seasoned. No plank to be wrought less than 24 Feet in length between the fore and after Shifts.  To have 3 Strakes between every 2 Butts on the same Timber, and none less than 6 Feet Scarph below the light Draft of Water may be either, English or Dantzig Grown Plank wrought 4 inches thick, the best of its kind, properly seasoned, but fresh & good, clear of Wain or any other Defect.  To leave at least 7 Strakes open next the Keel on each Side for Room for Chips and Water to come through, and not to stop the same, till near Launching, or that the upper Deck is caulked, and then the strakes remaining are to be brought on.

MAIN WALES …….	The Main Wales to be in Breadth, from Upper to Lower Edge 4 feet 3 inches, and in Thickness 8 ½  inches. To have one fair seam in the Middle & the two upper strakes to be locked into each other with Hook and Butt wrought of such lengths,& the Butts properly dispose so as to give the strongest shift to the Ports & to each other.

THICKSTUFF under    To have 6 Strakes of Thick stuff under the Main Wales: the Upper

MAIN WALES	 Edge of the Upper strake to be of 6¾ inches in Thickness the lower edge 				of the 3rd to be 5⅝ inches & the lower edge of the sixth Strake to be 4 				inches in Thickness.

THICKSTUFF on	To have One Strake upon the Main Wales of 6¾ inches in Thickness  MAIN WALES           and 1 foot 1 ½ inches broad, the lower edge of the Strake upon that to be

5 ¾ inches thickness & to wear off to 4 inches thick at the Channel Wales.

CHANNEL WALES  The Channel Wales to be in 3 Strakes of 2 feet 9 inches broad on the perpendicular and of an equal thickness of 5½ inches properly shifted & to work down to make a proper Stop and afford Wood for the Port Hook.

SHEER STRAKE       To have a Sheer Strake wrought on each side that its Upper Edge may be agreeable to the Upper Edge of the Waste Rail in the Waste, and one inch higher from thence forward and from the Waste aft to be in Breadth 12 inches and 4 inches thick of English Plank in Wake of the Channels.

STUFF between the    The first Strake upon the Channel Wale to be in thickness 4 inches & the

CHANNEL WALE    next under the Sheer Strake to be in thickness 3 inches & the intermediate

& SHEER STRAKE  	Strakes to be diminished agreeable to the lower and upper edges of these two.

SHEER  STRAKE      From the first Drift forward & aft to work one Strake of 3 inches thick on the Sheer Strake and the upper edge of the Strake under the Plankshire from the Drift aft to be diminished to the thickness of 2½ inches, which is to be dry English Plank.

IN HOLD LIMBER  The Limber Board to be 3 inches English Oak plank fitted as usual in His Majesty’s Yards.

STRAKE next the 	To have One Strake of 7½  inches thick, and 1 foot 2 inches broad next the

LIMBERS 	Limbers on each side, Distance from Side of the Keelson between the Magazine and Fish Room Bulkheads 11 inches, and from thence forward and aft to diminish to 5 inches & to have one strake, of  6 ½  inches thick, and 13 inches broad, wrought next to that, which 2 Strakes may be reduced to 5 inches in thickness.  Afore and Abaft that care be taken not withstanding the opening of the Strake next the Keelson diminishes the breadth of the Limber Board from the Fish Room aft & Magazine forward an opening for the run of the Water to the Pumps to be continued free and uninterrupted from the Gun Deck afore to the Gun Deck abaft & proper Limber boards fitted which may be taken up to clear the Space under them in Wake of the Magazine, Fish, and Bread Rooms.

THICKSTUFF at 	To have one Strake of 8¼ inches thick, and 1 foot 3 inches

FLOOR HEADS 	broad, wrought on the Joints of the Timbers at the Floor Heads, with one of 8¼ inches thick, and 13 inches broad and one of 6 ¾ inches in thick and 12 inches broad above and below it the Whole making 5 strakes which may be reduced to 4 strakes of 5 inches thick at the Fashion Piece and Apron.

THICKSTUFF at 1st	To have one strake of 7 inches thick, wrought on the joints of the

FUTTOCK HEADS   Timber at the First Futtock Heads, and both making 1 foot three inches broad, and One of 6 inches thick and 13 inches broad above and below it, which 3 strakes may be reduced to 2 strakes at the Fashion Piece and Apron of 4 inches thick, and 9 inches breadth.

FOOTWALING   	The Footwaling between the Limber Strakes and  Thickstuff at

between THICK 	Floor Heads to be 4 inches reduced to 3 inches afore and abaft.  The

STUFF	 Footwaling between the Thickstuff at the Floor and First Futtock Heads to be 4 inches English Plank reduced to 3 inches fore and aft

STUFF to be BOILED All Thickstuff both within and without Board, to be duly well seasoned sawn at least 8 months before it is used & that care be taken that all Plank or Thickstuff that is put into the Boiler, for the better bringing it about, be taken out whilst the Water is boiling. That no plank or Thickstuff be boiled but what necessity obliges.

ORLOP CLAMPS	To have 2 Strakes of Orlop Clamps on each Side, the Upper Strake 7¼  inches thick, and 1 foot 3 inches broad, the lower 6¼ inches thick, and   13 inches broad, which may be reduced to 1 strake at the Fashion Piece and Apron 4½  inches thick and 10 inches broad.

ORLOP BEAMS …..	To have Orlop Beams 1 foot 3½ inches square except the fore & aft beams which may be but 12 inches.  The second from Abaft 12 ¾ inches third 13½ inches 4th 14 inches fifth 14½ inches & sixth 1 foot 3¼ inches. The second from afore 13½ inches third Do, 14 ½ inches & 4th 1 foot 3 ¼ inches as those in Midships to be in number & stationed as directed by the Draught to be fayed to the Timbers to be kneed at each end with one Lodging Knee at each end fayed to the Timbers and one Standard Knee(except the 3 foremost & 4 after most Beams and to the sides of those Beams appointed for 2nd futtock Riders which are to have 1 Lodging Knee) both Lodging and Standard Knees to be sided 9¼ inches the Thwartships Arm  of the Lodging Knee to be 4 feet 6 inches long.  The fore & aft arm no to be shorter than 4 feet the thwartship Arm of the Standard Knee to be 4 feet & the up and down Arm to reach the lower edge of the upper Strake of the Gun Deck Clamp and to be bolted with 7   7 bolts of 1⅜ inches diameter.

ORLOP…… 	The Carlings & Ledges to be the same Scantlings as those on the Gun Deck and the Carlings to be disposed according to a plan that will attend this those that make the sides of Hatchways, & the tier next the side between the Platforms to be even with the Bean on the upper sides the other to be let down below the upperside of the Beam ½ inch. The flat of the Orlop between the Platform to be lay’d with Oak Board 1 ½ inches thick and in the wake of the  After Platform with 1 ½ inch deal, & the fore Platform over the Magazine with 2 ½ inch Plank, rabbetted. To have Stantions se up in the wings of the  Orlop 7 ½ inches square, properly tenon’d & chased into the Beam & Carling to confine the Cable: as also battens of 2 inches thick laid on the flat of the Orlop at 2 foot distance to keep it from the Wet, that the said Stantions but front up no further from the side than will just admit a Man passing Fore & Aft between them & the inside of the Gun Deck hanging Knees.

ORLOP CARLINGS	That the side tier of Carlings in Midships be of proper distance from the Ships side to receive the Stantions that form the Cable Tiers, which are just to give sufficient Room for passing behind them in the time of Action (but no more) as the breadth of the Coils of Cables should be reduced as little as possible, that those of the said side Tier of Carlings as come within the fore & after Platform be of a proper distance from the side for Scuttle being cut of dimensions proper for a person to go down under the said Platform in time of action in Wake of the Wing Bulkhead of the Magazine & Filling Room, which is to be fitted accordingly.

SAIL ROOMS	To have 2 Sail Rooms parted off in the Midships between the fore & Main Hatch of 1 ½ inch deal Rabbetted with a long Carling let down on the Orlop Beam of 12 inches deep & 10 inches broad to receive the Stantions of the said Sailrooms which are to be 7 inches square & the Deal wrought on the insides of them as shall be directed by the Draught as well as to their length and breadth.

STORE ROOMS	To have a Boatswain, Carpenters & Gunner’s Storerooms.  Block and Tar rooms, with a Fore Sail room on the fore Platform.  Steward room Captain Storeroom, Lieutenants Storeroom, Surgeon & Pursers Cabbins as also a Slop & Storeroom for Marine Clothing and on the after Platform with all such Conveniences both on the fore  & After platforms as are necessary & usually provided in Ships of War, and also to line & plaister such Bulkheads both above & below the said Platforms as shall be required for security of Powder, and as shall be directed by the proper officer.

IN HOLD ……	The Gun Deck Transom Knee to be sided 11 inches, the thwartship Arms to be feet 6 inches long. The fore and aft arm to reach the after Beam; Every other Transom below the Deck Transom to be kneed at each end with Knees sided 10 inches.  The thwartships Arms 5 feet long, & the Fore and Aft Arm 7 feet long or as shall be directed.

FLOOR RIDERS    	To have 3 floor riders, each to be in length 20 feet 6 inches, sided 1 foot 5 inches deep  on the Keelson and not less than 1 foot 4 inches moulded on the thickest Strakes at the floorhead 1 foot 2 inches & fastened with 12 Bolts of 1½ inch diameter.

LOWER FUTTOCK 	To have 6 lower futtock Riders on each side 1 foot, 4 inches, to be

RIDERS	 moulded on the thickest Strakes at the floorhead 1 foot 3 inches, the lower ends to reach within 3 foot of the side of the Keelson, & to be square at the end, not less than 6 inches, the heads to reach to the underside of the Orlop Beam, if can be got & moulded 12 inches there, & such of them as are placed by the side of floor riders to be fay’d and bolted with 3 fore and aft bolts of 1 ½ inch Diameter to each other.  To have Cross Chocks fay’d down on the heads of them/where there is no floor riders by the side of it/ with a Scarph of 6 feet in length wrought with hook and butt, the Chocks to be 13 inches on the Keelson and bolted with the same size Bolts of 1 ½ inch diameter as the Riders, except the two small Bolts in the Lip of the Scarph which are to be of ⅞ inch diameter.

SECOND FUTTOCK	To have 6 second futtock Riders on each side 1 foot, 2 inches sided, and in length from the floor rider head to the lower side of the Gun Deck Beam to be moulded of a sufficient bigness at the Orlop Beam & scored & tailed to the ends thereof with 2 Bolts of 1 ¼ inch diameter.  The Rider to be bolted with 9 in & out Bolts of 1⅜ inch diameter, the upper part of the lower futtocks, & lower part of the 2nd futtocks to be fay’d &  bolted sideways to each other with 2 bolts of 1 ½ inch diameter.

CRUTCHES ……	To have 2 Crutches in the Run Abaft, one in the place of the Mizen Step, sided 13 inches. The Arms to be in Length 8 feet, and bolted with 8 bolts of 1⅜ inch diameter.

MIZEN STEP …..	The Mizen Step to be 1 foot 8 inches of sufficient Depth on the Keelson, the Arms  to be 7 long each, and bolted with 12 bolts of 1⅜ inch diameter.

MAIN STEP …..	The Main step to be sided 2 feet 5 inches and 1 foot 5 inches deep on the Keelson, & of a length to slide easy by the Stantions of the Well.

FORE STEP …..	The Fore Step to be made of 2 Crutches each to be sided 11  inches & 3 inches asunder in the clear, equally Distant from the Centre of the Mast, & in length 12 feet with Carlings & shifting Chocks fitted to them as shall be Directed & bolted with 8 Bolts in each of the 1¼ inches diameter.

BREAST HOOKS ..	To have 4 Breast Hooks below the Gun Deck Breast Hook, and equally divided between that and the Fore Step, the Upper one to be in Length 19 feet 0 inches, the Lower one 15 feet 6 inches, to be sided 14 inches, and bolted, the upper one with 13 and the Lower one with 11 Bolts of

1¾  inches diameter.

PILLARS in HOLD	The Pillars in Hold under the Gun Deck and Orlop Beams to be 9¾  inches square at the lower end, and 8¾ inches at the upper end.

WELL …..	To have a Well 10 feet Fore and Aft, 8 feet thwartships or as the Draught shall direct, birthed up with English Plank of 3 inches thick from the Orlop down, and from thence up with Weather Boards, wrought Looverways.  To have Doors, &c. as usual.

SHOTLOCKERS	To have  Shot Lockers afore, and another abaft the Well, of the same Breadth athwartships as the Well, and in the clear Fore and Aft to be 2 feet 2 inches, the Sides of the Well to run forward& aft  sufficient to receive the ends of the Lockers & a Cant to keep the Nails from Drawing by the weight of the Shot, To have a Partition in the Middle of the said Lockers, with Lids hung as usual, and another Locker abaft the Magazine between the after pair of riding Bitts.

MAGAZINE …..	To part off & build a Bulkhead for a Magazine with 3 inches Oak Plank rabbetted. The Stantions to be 6 ½ inches square and 2 feet 8 inches asunder, the said Bulkhead to have its Seams lined on the Aft Side with slit Deal battens,  plaistered and Cant, & plaister the Sides & Bottom, the foreside to be plaistered wholly over & Covered with Deal of 1 ¼ inch thick & canted to break the joints as usual, & between the Ledges on the underside of the Platform. and over the Magazine it is to be also plaistered, and lined with battens to break the Joints.   To have  a Double Palleting made with Scuttles on the flat and cants laid thereon proper for the Powder Barrels to Stow & to be kept free from Wet, & Wing Bulkhead fore and aft at the side with Pannels to take up and down, to be lined with feather edged Deal at the Ships Side & the whole Magazine completed with everything necessary.

FILLING ROOM	To part off  Filling Rooms, plaister and single line the same on the Outside, and plaister and double line the same within, the first a Slit Deal Lining upon the Mortar as usual, and on the Lining to bring on a dry Lining fastened with Brads punched up, and their Heads putty’d all round where filled Cartridges are kept; to provide proper shelves for stowing the same, with shifting Battens, Drawers, &c,  as is usually done on board ships of her Class in the King’s Yard.  That the floor of the passage to the said filling room & Magazine from the first Door next to the filling Room, out to the inner side of the Door or Cill just without the Scuttle on the Dun Deck thro’ which the Powder is taken in & carried out, to be covered with Lead 5 pounds to the foot square, and tuned up all round against the Sides at least 5 inches & so carefully done as to retain water therein in time of action, & with Cills of at least 7 ½ inches deep in the said passage to prevent any accident from the quantity of Waste Powder that may be spilt therein.

FOREPEEK &C. 	To make all proper Lockers in the Forepeak for the Gunner, & also in the Carpenter’s & Boatswain’s Storeroom, with such Shelves, Bins & other Necessaries as those are usually fitted with in Ships of War, to line the upperside of the Floor of the Platform with Plaister covered with solid Deal, and to build up and line as shall be directed all Bulkheads for Passages to the Magazine and Lightroom, and also to line the Lightroom with Tin Plates where shall be directed, put up proper partners for the Powder room Lanterns, and provide a Sash of stone ground Glass with Copper Hinges to be hung to the Beam for letting down or turning up for a Security to the light, and do everything on the said Platforms as shall be directed by the Officers, or Overseers appointed to inspect the same.

SPIRITUOUS 	To build a Bulkhead for the Spirituous Liquor Room with 3 inch

LIQUOR ROOM	 Plank ciphered or rabbitted; To battin and cant the same on the Foreside with Plaister and battins to cover the Joints, and to wholly cover and line the said Bulkhead on the Aft Side with Plaister and Slit Deal: make Hatches, and do what is required for the Security to keep Malt Spirits confined therein.

FISH and BREAD	To build Fish and Bread Room Bulkheads, line the Bread Room,

ROOMS 	& build an after Powder Room and furnished with Shelves and Lockers, and a Light Room  properly lined and to complete the whole work both upon and under After Platform agreeable to Ships of the Class, as shall be directed by the Draught for carrying on the same, and that all the whole Slit and Feather-edged Deal used on these Works be yellow Wood, well seasoned and wrought, clear of Sap, and the outer Surface planed, that it may not retain Dust &c.

GUN DECK 		To have 3 Strakes of Gun Deck Clamps on each Side the upper  to be

CLAMPS		8¼ inches thick and 1 foot 5 inches broad, the second Strake 7 ¼ 					inches thick and 1 foot 2 inches broad, tabled on into the other with 				Top and Butt, the Upper Strake to be scarphed with Scarphs 3 feet 6 				inches long, and the 3 Strakes to be reduced to 2 inches at the Apron and 				Fashion Piece and in the thickness to 5 inches preserving their proper 				thickness to within about 5 feet of the extreme.

STRAKE on the ENDS	To have one Strake of 6¼  inches in thickness, and 1 foot 1 inches broad,

of ORLOP BEAMS 	wrought on the Ends of the Orlop Beams, and that the opening between the lower edge of the Gun Deck Clamps and upper edge of the Strake upon the Orlop Beams be shut in with 4 inches Oak Plank.

GUN DECK 	The Gun Deck Breast Hook, and that under the Hawses to be sided

1 foot 2  inches, and in Length 18 feet 6 inches, bolted with 13  Bolts of 1⅜ inch diameter. The Deck breast Hook to be fayed to the Timbers, and to be moulded as broad as sufficiently may be for better fastening the Plank of the Deck.

GUN  DECK 		The Gun Deck Beams to be moulded 1 foot 4½  inches, and sided                         BEAMS		1 foot 4½  inches; the  fore and aftermost to be 2 inches less, the second

1½ inches, the third 1 inch, and the fourth from afore and abaft ½ inch

less each way,  to be in Number, and disposed as in the Draught appointed herewith and in the wake of the Main Hatch, and Mast, or where the Arm of a Beam is wanting to support the Gun or receive a Standard have double Armed Beams.  The arms to be 1 foot 3 inches deep and 14 inches broad, that the Beams are sound, free from Shakes of any Kind, and those that are made in Two Pieces, the Scarphs for the longest Beams not to be less than 12 feet in Length, carefully tabled one into the other, the Ends to be left 3 inches thick when wrought and the Scarphs bolted with 8 bolts 1⅛ inches diameter or more if thought necessary.  And as it has been directed that the Gun Deck Clamps should work up to the underside of the Lodging Knees, which tho’ they are kept below the Beams 1½ inches the Beams would cut the Clamp too much and therefore it will be proper to cause the said gun Deck Beams to be bearded back at least 3 inches on the underside that they may not cut above 2 or 2 ½ inches down on the oppe3r edge of the Clamp. – And as the Beams require to be well seasoned, they should be converted as soon as possible, scarphed and laid in Parts as long as the Works will admit before they are bolted, when tar and Hair are to be laid between the Scarphs, and when cut of a length,  hopes are to be bored in the pith and the Ends burnt as is the practice in His Majesty’s Yards, the upper part of the Scarphs to be properly caulked.

GUN DECK	The Gun Deck Beams to be kneed with one hanging and one lodging Knee at each end,  with one the  hanging and one lodging Knee; the hanging Knee to be sided 10 inches, the up and down Arm 6 feet 2 inches long; The Thwartships Arms 4 feet 6 inches, bolted with 8 bolts of 1⅜ inches diameter except the five fore and after Knees which may have but  7 in No. viz, where 8 bolts 5 of them to be in the hanging arm and where 7 Bolts, 4 to be in the hanging Arm, and to have hanging Knees of Iron to the two after Beams, as wooden ones cannot be got of a proper growth for the purpose, the Throat Bolt and next below it, to be placed as high as may be, and all carefully forelocked and the up and down Arm of these and the Standard Knees to the Orlop Beam to be disposed so as to be clear of each other.;

GUN DECK 		The Lodging Knees of the Gun deck to be sided 9½ inches; The LODGING KNEES   Arm to the Side the whole Length between the Beams, or sufficient

for 4 Bolts where Distance between the Beams will admit, and the Thwartships Arm to be 5 feet long, bolted with 7 Bolts of 1⅜ inches diameter. All the Transoms, Breast Hooks, and Gun Deck Knee Bolts to be well forelocked.

CARLINGS and 	To have 3 tiers of Oak Carlings on each side from the Room before the

LEDGES 	Fore Hatch, to the Room of the Mizen Mast from thence forward and aft to have 2 Tier of Carlings, to be  9 inches broad, and 8½ inches deep..  The ledges to be   spaced not more the 12 inches, nor less than 9 inches asunder, 5½ inches broad and 5 inches deep.

BITTS PINS		To have 2 pair of Riding Bitts their Heads to be above the Deck 5 feet 				3 inches the foremost	Pair to be Abaft the After part of the Stem 29 feet 				6 inches, or as the Draught shall direct, the Aftermost to be abaft the 				foreside of the foremast 11 feet. Distance between, them Athwartships 				Foremost 3 feet 8 inches; the Aftermost to be 4  feet 4 inches to be square 				from 12 inches under the Gun Deck Beam, up 1 foot 8  inches the 					After ones, and 1 foot 7½ inches the Fore ones and to taper from thence to 				12 inches at the lower End: To be let forward with a score of 2½ inches 				under the Gun deck Beam and bolted with 2 bolts 1⅛ inches diameter.  To 				have large cleats against the foresides of the bitts under the Gun Deck 				Beam, of 7 inches thick 12 inches broad, and 4 feet 6 inches in length or 				to run down to the Orlop Beam, and bolted and nailed to the bitt Pins to 				prevent the said beam from being forced out of its Score by the Strain				of the Bitt Pin which is too often the Case if not guarded against.

CROSS PIECES 	The Cross Pieces to be fore and aft 6 feet 6 ½ inches, deep 6 feet 5 inches. THE BITTS		To have an Elm Back of 6 inches in Thickness, added to each Cross

Piece, be above the Deck, the Foremost 1foot 9 inches, the Aftermost 1foot 8 inches, scored 2½  inches on, and fitted with proper Hooks and Eyes to the Bitts. The ends without the Bitts 2 feet 3 inches.

STANDARDS to the	The Standards to the Bitts to be sided 13 Inches to be let down below the BITTS			Gun Deck Beam 1½  inches on broad Carlings prepared for them, the 					Foremost ones to run to the Beam before the Fore Mast, and with the 					Carling below make the outer Piece of Partners, the aftermost to be 					continued to the aft side of the foremost Bitts, and both of sufficient Depth 				on the beam, and well bolted through the Gun Deck Beams and Carlings, 					with Bolts of 1¼  inch diameter.  The Upper Part of the Up and Down 					Arm to be equal to the Upper Side of the Cross Piece.

GUN DECK            	The Lower Deck Waterways to be well season Oak Plank of 5½ inches

WATERWAYS	thick, chined down to 4 inches, the thickness of the Deck, to be bearded back ⅜ of an inch, and to be left loose so long as the Works will admit to give the Ship Air.

WING TRANSOM 	To have two Wing Transom Knees sided 12 inches the fore and aft arm to be 16 feet long, the thwartships arm to be 6 feet long, to Scarph with Hook & Butt upon the upper Strake of the Spirketting and bolted with 5 bolts of 1 ¼ inch diameter in the thwartships arm and with 8 bolts in the fore and aft arm, and with 2 Bolts of ⅞ inch in the lip of the Scarph.

FLAT of the DECK	The Flat of the Gun Deck 5 Strakes next the Waterways, and all the Foreship to be of English Plank.  To have 2 Strakes of 5 inches thick scored down one inch on the Beams and Ledges, the First to be the second Strake next the Comings, the other to be the third, to give proper Shift to the Comings, Partners, & Strake cut off by the Pumps as well as to each other, To be fastened with one ⅝ inch bolt and one Nail in each Beam and one three nail in each Ledge, the rest of the Deck to be either English or the best East Country Crown Plank fastened with 2 nails in each beam & one tree nail in each Ledge.

MAIN PARTNERS	To have two Carlings for the Partners of the Main Mast 1 foot 5 inches broad, and 1 foot 6  inches deep.  To be let down Carling Fashion, 3 feet 9 inches asunder in the clear; to be above the Gun Deck Beam 8½ inches;  and the Lap to be continued from the Fore to the Aft sides of the Beams, and to have proper Rabbits to receive the Cross Chocks of 9 inches thick, bolted through the Beams with Bolts of 1⅛ inches diameter.

FORE PARTNERS  	The Fore Partners to be made by Carlings of 1foot 4 inches broad, and 10 inches deep, let down 1 ½ inches below the Beam, and 3 feet 6 inches asunder.  The Standards to the foremast Bitts to make the upper part of the Carling and form the Rabbit for the Cross Chocks, which are to be 8 inches thick as before observed.

MIZEN PARTNERS	The Mizen Partners to be 5 inches thick and 4 feet in breadth.

STEP OF THE 	To have a Step for the Bowsprit in 2 pieces, rabbetted into each other of BOW SPRIT 		4 feet  2 inches in breadth & 1 foot 1 inch in thickness, to be in length 				from the upperside of the upper Deck to the lower side of the Gun Deck 				beam, let aft 2½ inches on each & bolted with Bolts of 1¼ Diameter.

STEP FOR THE	The step for the Jeer Capstand to be prepared to shift for the capstand

JEER CAPSTAND	being lowered down to stow on the Orlop out of the way of the Long Boat, as the Ships lately fitted in His Majesty’s Yard or as shall be directed.

STEP FOR THE 	To have a Step fixed and let down Carling fashion for the Main Jeer

MAIN CAPSTAND 	Capstand 1 foot 8 inches deep and 1 foot 9 inches broad secured with Plates and Bolts to prevent the wood on the lower side from splitting by the fall of the Spindle, on putting under the <Vyol.>XXXX

HATCHWAYS	The Main Hatch to be Fore and Aft 8 feet 4 inches, and thwartships 6 feet 4 inches.  The Fore Hatch to be Fore and Aft 4 feet 9 inches, and thwartships 4 feet 8 inches. The Hatch Abaft the Main Mast, Fore and Aft 4 feet 8 inches, and thwartships 4 feet 8 inches.  The Hatch afore the Fishroom and the Fishroom hatch, Fore and Aft, 4 feet 8 inches, and thwartships 4 feet 8 inches.  The Hatch to the Steward’s Room Fore and Aft 3 feet 10 inches, and thwartships 4 feet 8 inches.

COMINGS and HEAD  The Comings to the Hatchways on the Gun Deck to be 3½  inches above

LEDGES 	the Deck, and to be wrought down to the Beams and Carlings.

GRATINGS	All the Hatchways to have substantial Gratings with a Carling of  9 by 7 inches, let down under the middle of them.

LEAD SCUPPERS	To have 6 Lead Scuppers on each side 4 inches diameter, in the clear, well let out and turned, so that they shall be perfectly tight round the flap.

MANGER	The Manger Boards to be 4 inches thick rabbetted, and well secured with cant on the aft side.	To have 2 Lead Scuppers in the Manger on each side 5 inches Diameter in the clear, and two Pump Dale Scuppers 6½ inch diameter in the clear, and if but one, to be of a large Diameter to let the Water, as shall be required.  The Pump Dale Scuppers to have flap of 2 ½ inches broad on the inside and that let in 2 inches into the Spirketting to receive the end of the Pump Dale.

PORT CELLS	The Gun Deck lower Port Cells to be 7 ½ inches deep. The upper Cells in Wake of the fore & main Channels to be 10 inches deep to receive a preventer Bolt if required.  The Cells to be put in as is done in the King’s Yards.

SPIRKETTING	To have 2 Strakes of Spirketting on each side 6 inches thick, wrought with Hook and as is done in the King’s Yards.  To have a 1 inch diameter Bolt next to the Butt of each piece and to that in between the Spirketting & Upper Deck Clamp with 3 inch Oak Plank, that great care be taken to shift the said Spirketting clear of the Ports, and to give Scarph to each other, and of such lengths, that in general each piece may over launch 3 Gun Deck Ports; That the Butts are properly disposed  for the Pump Dales, that it be well seasoned, and therefore should be cut, at least 8 months before it is used.

GUN DECK PORTS 	To have 14 Gun Deck Ports on each side fore & aft 3 feet 5 inches, except the foremost which may be 3 feet 3 inches by lines across, and the aftermost 3 feet 4 inches & 2 feet 8 inches deep, & to have one port in the Counter on each side thwartships 2 feet 5 inches & deep 2 feet 3 inches.

PILLARS	The Pillars under the Upper Deck Beams to be 8½ inches square at the lower end, and 7½ inches at the Upper End, and turned.

STANDARDS	To have 12 Standards on each side on the Gun Deck, the Wood   sided 12 ¼  inches; of iron, The foremost pair and as many of the others to be Iron as shall be directed, the up & down  arm to reach the upper edge of the upper deck Clamp; the other 4 feet 3 inches long, bolted with Bolts in each of 1½ inches diameter.  The Standards, both Wood and Iron, to be all fayed on Shoals of 3 inches.  Plank laid with tar & Hair on old canvas. To have a Standard on the Gundeck against the head of the Stern Post, sided 13 inches, the fore & aft Arm to be of sufficient length to receive a Bolt in the 3rd gundeck Beam from Abaft, to be scored down on the Gun Deck Beam 2½ inches the up and down arm to reach the upper side of the Helm port Transom, & to have a Carling from the Beam abaft the Mizen Mast to the Transom 13 inches sided, scored up between the Beams, and that the space from the lower side of the Knee to the upperside of the Carling be filled up by fillings let down between the Beams, before the Knee is fayed that the said Standard may be bolted by one Bolt in each Beam, and one Bolt between each Beam, by Bolts of 1¼ inch in diameter and to be well clenched under the said Carling.  To have a Standard on the Gun deck against the Apron sided 13 inches the up & down Arm 5 feet 9 inches long. The other to be long enough to receive a Bolt in the 2nd  Beam from afore and to be properly secured.

TILLAR	The Tillar to be clear of Knots 11⅜  inches square in the biggest Place, which is to be continued 2 feet before the Rotherhead, To have a Strap of Iron round the After End, of sufficient Length, let into the Wood, and bolted with Bolts afore the Rother Head, and to have a Hoop, and Eye on the Fore End for the Rope: the Sweep to be 3 inches thick, flush with the Underside of the Beams, and 11 inches broad, guarded with an Iron Plate on the Upper side, and secured with proper Straps in the Quarters.  And fitted with rolls as is done in the King’s Yards; To have Blocks or Eye Bolts for the Tiller Rope at the Sides, as also under the Deck, as shall be required.

HELM PORT 	The Helm Port Transom to be 10 inches deep scored& bolted through

TRANSOM	the Counter Timbers to give it no more cast than necessary to give room  for the tillar which is to have 3 inches play above & below, between the 	  Deck and Helm Port/and Stern Ports.  The Knees sided 9 inches.  The fore & aft Arm 10 feet 6 inches, the other 5 feet 8 inches long, bolted with the same sized Bolts as the upper deck Knees, and to let on upon the hanging Knees of the upper deck which come in Wake thereof, and as the said Knee should be well grown and have little Chock, if any, the Hanging Knees are to be of Iron, that the letting them into the fore & aft Arm of the Transom Knee may not cut it too much and in the Cast part of the Transom, where the Wood is apt to spring to have two small Bolts drove down from the upperside, and clenched on the underside as close as may be, to the middle part on which the Tillar traverses.

RING AND EYE	To have 4 Ring & 2 Eye Bolts to each Gun Port with Bolts of 1⅜ inch

BOLTS 	diameter; 2 of the Ring Bolts to be placed in the second Timber from the Port, the rings 5 inches diameter in the Clear the Eyes sufficiently open for the Tackle Hooks.  To have a sufficient number of Ring Bolts on the Deck for the Guns with Bolts of 1⅛ inch diameter; and for the Stoppers with Bolts of 1⅝ inches diameter, the diameter of the Rings o the Stopper Bolts 6½ inches in the clear with short snug eyes let well down into the Deck & the ends of the bolts to be all Clenched.

EYE BOLTS	To have 2 Eye Bolts over each Port for lashing the Guns of 1 ¼ inches diameter & to be place as high as possible to give the better room for securing the Muzzle of the Gun on the Clamp. The Eyes to be 2⅝ inches in the clear, the ring & Eye Bolts to be well forelocked & so will let in as only to let the ring have play, the Bolts that Forelock without Board, to have short thick Points, and Forelock Holes that the Rings may be let into the Wood in belaying them.

PUMP CISTERNS	To have 2 Pump Cisterns, one abaft the Main Mast, the other afore it as shewn on the Draught to be from out to outside  2 feet & 2 feet 7 inches deep, height from the Deck 1 foot, the Plank for the Bottom to be 4 inches Oak,  for the Ends to be 5 inches Oak & for the sides 3 inches Oak Plank the Ends of the Cisterns to be 8 inches without the Heads of the Pumps.  To have 2 partitions in the Middle of 3 inches thick & of ¼ inch asunder that on Occasion a Pan being run between them may make them independent of each other with Scuppers &c.

HAMMOCK RACKS  To have all necessary Hammocks Racks & Tracing Battens to lie properly fastened to the sides of the upper Deck Beams & the tracing Battens to the upper Deck Carlings.

CENTER of MASTS	The center of the Fore Mast to be 18 feet 6 inches abaft the aft side of the stem and to rake Aft ⅛ of an inch in a Yard.  The Center of the Main Mast 94 feet and 5 inches abaft the aft side of the Rabbet  the Stem to rake aft 1 inch in a Yard.  The Center of the Mizen Mast 26 feet 8 inches afore the Rabbit of the Stern Post on the Gun Deck, and to rake Aft 1⅜ inches in a Yard.

UPPER DECK 	The upper deck Clamps to be 5 ¼ inches at the upper edge bearded from

CLAMPS	 half its Depth to 4 ½ inches at the lower Edge, the Scarphs to be 4 feet 6 inches long wrought with hook & Butt, and Bolted up & down through the tops with 2 bolts of ⅞ inches diameter

UPPER DECK 	The Upper Deck Transom to be 10 inches deep, scored and bolted TRANSOM 		 to the Counter Timbers, and kneed as the Deck, with one Lodging

Knee at each End.

UPPER DECK 	The Upper Deck Breast Hook 13 inches sided, in Length 18 Feet 0

BREAST HOOK        inches, bolted	with 13 Bolts of 1¼ inch diameter.

UPPER DECK 	The Upper Deck Beams to round 7¾  inches, to be sided 14 inches,

BEAMS                      and moulded 12 inches. To have One under and One between each

Port, and as near as may be over the Gun Deck Beams, or as the Draught shall direct: and in the Main Hatch and Mast Room to have double armed Beams, if needful, so that one of them may be proper for a Standard, and the other under the Port. The Beams to be sound, free from Shakes, and well seasoned, those that are in two Parts, the Scarphs to be from 11 feet 6 inches in the greatest lengths, & One-third of the Beam for the rest, to be wrought Tables whose Butts or Squares are to meet & correspond with one another, in the most exact manner avoiding hollow work, & that each Scarph bolted with 8 bolts by 1 inch diameter, & that the works on the said Scarphs be every way equal to what is done in His Majesty’s Yards & as the should be well seasoned the said Beams should be converted & wrought sometime before they are wanted, but the Scarphs are not to be bolted until just before, or after they are crossed.

UPPER DECK 	The Upper Deck Beams to be kneed at each End with one Hanging

HANGING KNEES  and one Lodging Knee, the Hanging Knee sided 8½ inches, the Up and Down Arm to reach 6 inches on the Spirketting, the other to be 3 feet 9 inches long, bolted with 8 bolts in each of those hanging Knees between,  the Fore Hatch & Mizen Mast, 5 of which to be in the Hanging Arm & in the Knee afore and abaft these to have 7 bolts of which four to be in the hanging Arm, with Bolts of 1⅛ inch diameter.  The throat Bolt & those below them, to be placed as high up as may be & that those to the two after Beams to be of Iron, that by being let into the Arm of the Helm Port Transom Knee, they may not wound the same.

UPPER DECK	The Lodging Knees to be sided 7¾  inches, the Arm next the Side the

LODGING KNEES     whole Length between the Beams, or sufficient for 3 Bolts, or more if the length requires, and that to the beam 4 feet 2 inches long, at least, bolted with 3 bolts of ⅛ inch diameter, all well clenched.  No Openings to be cut under the Lodging  Knees.

CARLING under the 	To have a long Carling under the upper deck Beams 12 inches broad &

FURNACES	 12 inches deep of which 10 inches to be whole Wood under the upper deck Beam, or exclusive of 2 inches it is to be let up between the Beams & furred from thence to the underside of the Ledges.  The Carlings to be in length from the fore side of the Beam abaft the Foremasts to the aftside of the Beam, at the foreside of the Fore hatch, & properly supported with Pillars.

CARLINGS and 	To have 3 Tier of  Fir Carlings on each Side from the Fore Hatch to the

LEDGES 	Mizen Mast room, from thence forward  to have 2 Tiers, & in Wake of the Fire Place & Main Hatchway to be of Oak. The Carlings to be 8 inches broad and 6 inches deep, the Ledges 4 ½ inches broad & 4 inches deep, not less than 9 inches nor more than 12 inches asunder. The Ledges also to be Oak in Wake of the Oak Plank next the Side, & from the Mizen Mast aft to have half Beams to be let into Oak Carlings of sufficient Distance from the Side to Shift the knees, the Half Beans to be of Fir 10 inches broad & 7 ½ inches deep, and the Carlings at the side which they are to be let into to be 8 ½ inches broad and 9 inches deep.

COMING CARLINGS	To have long Coming Carlings of Oak on the Midship Carlings 11 inches broad, scored 1 inch on  the Beams and Ledges; to rise 3 ½  inches above the Deck, bolted with 2 Bolts in each Beam of ¾  inches diameter: To be of such lengths as to give shift to the work on the Deck & agreeable to the range of Gratings, enclosed with head Ledges wrought down on the Beam of an agreeable round, but the Grating to be divided or parted by only Fir Ledges of 5 inches broad.

UPPER DECK 	To have substantial Gratings neatly made & plained & the Ledges struck

GRATINGS	with a Bead, with a Fir Carling 7½ inches deep by 5½ inches broad under them.

UPPER DECK 	The Waterways to be 4½ inches & the Deck 3 inches thick. The foremost

WATERWAYS 	lengths & as far out as the Standards to be English Oak Plank, to have 2 Strakes of Oak Plank, the 2nd and 3rd next the Comings, on each side Scored down on inch, the rest of the Deck laid with 3 inches Prussia Deals, and well fastened to the Transoms, Breast Hooks, Beams, and Ledges.

MAIN PARTNERS	To have Partners to the Main Mast 3 inches thick, and Prussian Deal laid on Ledges, and in a Rabbit prepared in the Comings and Head Ledges.

FORE PARTNERS	The Carlings for the Foremast to be 14 inches broad, and 15 inches deep, let down  on the Gun Deck & formed with Cross Chocks I the same manner the Carlings to be the same Distance, as those below, and to be high above the Deck 4½ inches.

BLOCKS	To have Taylor’s Patent Brass Coaks, & Iron Pins for the Main Sheets, & all the Blocks in the Ship.

PARTNERS for	The Partners for the Jeer and Main Jeer Capstands to be 7 inches thick &

the CAPSTAND	 filled as is done in the King’s Yards for ships of her Class, that the Fore Jeer may be lowered down out of the way of the Long Boat.

MIZEN PARTNERS	The Partners of the Mizen Mast to be 5 inches thick, and of proper Breadths scored down 1 inch, & fastened with Bolts of ⅝ inches diameter

CAPSTANDS	To make and set a Jeer Capstand afore & a Main Jeer Capstans  Abaft on    the Upper Deck,   1 feet 11 inches diameter in the Partners, each fitted with 12 Ash Bars of 12 feet 6 inches long each, with ribs and Hoops in the Partners, & Hoops Sole & Bolts in the Step, Cranks for the Bars, Iron Pins and Chains, and 4 Iron Pawls to each, Two of which to fall on a Trundle Head, and to answer the Pawls on the Deck, & in every respect to compleated as is done in His Majesty’s Yards.

PORT SILLS	The Upper Deck Port Sills to be 6½ inches deep, put in with a Bill, as is done in His Majesty’s Yards.

SPIRKETTING	To have 2 Strakes of Spirketting on each Side at the Lower Edge 5 inches & the upper edge 4 inches thick & to shut I from the Spirketting to the String between the Ports, with 2 ½ inch Prussia Deal or Oak, the ends turned off with a Quarter Round, as also the Upper and Lower Edges of the String and Spirketting.

TRANSOM	To have a Transom cross the Stern at the Height of the Ports, 7 inches deep, scored and bolted to the Stern Timbers, and kneed at each End with One Iron Knee, which is to be cast down under the Gallery Door, and to be continued long enough to receive 3 bolts afore it.  The Thwartships Arm to be 4 feet  & bolted with saucer headed Bolts of 1⅛ inches diameter, well clenched without-side.

COLLAR BEAMS 	To have a Collar Beam afore at the heights of the Ports 11 inches square kneed at each end with one substantial Iron Knee, bolted with six Saucer headed Bolts of 1 inches diameter, or have 2 Stantions run down to a Cant on the upper Deck & half Beams, with a Rabbet of the Plank of the Beakhead as shall be directed.

UPPER DECK PORTS	To have 14 Upper Deck Ports on each Side, as shown on the Draught, Fore and Aft 	2 feet 10 inches, and in Depth 2 feet 8 inches, 2 Ports in the Stern, and two in the Beakhead, with proper ring and Eye Bolts for the chase Guns of the forecastle, with proper Doors & places for easement for the Officers, & to make and hang all such Port Lids, as shall be required& fit them with proper shackles for hawling them up, & baring them in, & also to make all such half Ports, with whole deal  fayed to the Stops& to line the same with slit Deal as usual, and shall be demanded on the upper Deck.

BOATSWAIN &	To have a cabbin for the Boatswain & Carpenter at the Bulkhead of the

CARPENTER	 Forecastle, as usual.  To have blocks with live shivers let through the side

CABBINS & MAIN 	against the  Mizen Mast on the Upper or Quarter Deck, for the Main

SHEET BLOCKS         Sheets, as also others properly placed for the fore & sprit sail sheets, & all fast Blocks, ranges & Kevels &c. as is done in His Majesty’s Yard to Ships of her Class.

LEAD SCUPPERS	To have 7 Lead Scuppers on each side, or as shall be directed, 4 inches diameter in the clear, carefully let out & turned, that no leakage be found on the flaps thereof.

STANDARDS	To have 12 Standard on each side on the upper Deck, those of Wood sided 10¾ inches as many of them Iron as shall be directed, and of the usual weights to ships of her Class.  The up & down Arm to reach the upper edge of the Quarter Deck or Fore Castle Clamps, or String of the Waste, the other of Iron 3 feet 4 inches, if Wood 3 feet 9 inches long, bolted with 7 Bolts in each, of 1⅛ inch diameter.

RING and EYE 	To have 2 Ring and 2 Eye Bolts to each Upper Deck Port of 1⅛ inches

BOLTS 	diameter the Rings 4¾ inches diameter in the Clear, with a sufficient number of Ring Bolts on the Deck of 1⅛ inches diameter, and 3¾ inches in the Clear for hauling in the Upper Deck Guns those through the Side to be carefully forelocked and those through the Deck clenched under the Beam.

STRING in the	The String in the Waste to be 4 inch Plank, to give good Scarph to the

WASTE 	Drift and sheer Strake, and to be locked by Hook and Butt into the Forecastle and Quarter Deck Clamp; The Planshires to be 3 inches Plank, wrought with a long Scarph, hooked and bolted with 3 Bolts of  ⅞ inches diameter; those in the Drifts to be 2 feet 2 inches Plank, struck with an Astrigal projecting the Work within and without board.  The String of the Waste to be also wrought with a long Scarph between the Ports & the Sheer Strake, without Board, & the String within to be securely bolted with Bolts of ⅝ diameter, to every 3 feet Distance, proper Fillings having been wrought between the Timbers in wake thereof.

MAIN JEER and TOP	The Main Jeer and Topsail Sheet Bitts to be 13 inches Square from the

SAIL SHEET BITTS 	Lower Side of the Upper Deck Beam to 2 feet 6 inches above the Deck, and the Jeer   Bitts to run up and tenon into the Beam of the Quarter Deck. The Cross Piece to the Jeer Bitts 8½ inches deep by 10 inches broad,   , scored 1½  inches on the Bitt Pins: The Top said sheet Bitts to tenon into the fore Beam of the Quarter Deck.

BULKHEAD to 	To have a bulkhead to the Wardroom, with such Cabbins as Ships of her

WARDROOM 	Class are usually allowed, according to the Establishment of the Navy, or as shall be directed by the Plans, or Officers appointed to inspect into the same.

EYE BOLTS for	To have 4 Eye Bolts properly placed for the Toptackles of 1⅝ inches

TOPTACKLES 	diameter and proper Eye Bolts on the Deck about the Mast for lashing Blocks of 1⅛ inch diameter.  That great care be taken that the Eye Bolts for the Toptackles b forelocked with a thick forelock on a square Plate, and be wrought with great care, out of the best sort of Orground Iron.

QUARTER DECK	The Quarter Deck Clamps to be 2 Strakes, 4 ¼ inches thick at upper edge,

CLAMPS	beard to 3 ¼ inches at the lower edge of the upper Cill of the upper Deck Ports, where the lower Strake of Clamp is to work & answer to the String of the Waste.

QUARTER DECK 	The Quarter Deck Beams to round 8½ inches, to be sided 9½ inches, and

BEAMS	 moulded 8 inches, placed as on the Draught, and to be cut out of clear grained young Oak Timber, well seasoned.

PILLAR UNDER the	To have Pillars to the Quarter Deck, Beams, handsomely turn 6 inches

BEAMS	 square at the lower end & 5 ¼ inches at the upper,  To have Iron Pillars under the Quarter Deck in Wake of the Main Capstand, and under the Forecastle in Wake of the Fireplace, not less than 2 ¼ inches square.

QUARTER DECK 	The Quarter Deck Transom to be 7½  inches deep, scored and bolted to the

TRANSOM	 stern Timbers, and kneed at each End with one Iron Knee of sufficient Length to have two Bolts afore the Gallery Door.

QUARTER DECK	The Beams of the Quarter Deck to be kneed at each End with one

KNEES 	Hanging and one Lodging Knee the Hanging Knee sided 6½ inches: the Up and Down Arm to reach the Spirketting, the other to be 3 feet 2 inches long, bolted with 8 bolts of ⅞ inches diameter. The Lodging Knees sided 6 inches, the Arm next the Side the whole Length between the Beams the other 3 feet 6 inches long, bolted with 5 bolts of  ⅝ inches diameter.

WATERWAYS	The Quarter deck Waterways to be 4½ inches Plank, and 2 Strakes of English Oak next them; the rest of the Deck to be laid with 3 inches Prussia Deal, and to have a balcony projecting the stern, as on the Draught, the Deals to be well seasoned, clear of Sap& large Knots, & not to be wrought broader at the fore end than 10 inches.

STERN PORT and	To have Stern Ports through the screen Bulkheads the Sashes to be in one

SCREEN BULKHEAD    thickness and of a Depth proper to give light to the great Cabbin, and that the Pannel below be no deeper than just sufficient to bury the Glass part of the sash, to have substantial stiles of 2 inches Planks fitted with Ring Bolts for using the Guns right aft, the whole Bulkhead and Walk to be fitter as is done in His Majesty’s Yard or as shall be directed by the draughts prepared for the doing thereof.

QUARTER DECK	The Gratings on the Quarter Deck to be of 3 feet 8 inches broad in the

GRATINGS 	clear afore and ¾ inches abaft, or a the plan shall direct,   handsomely made as usual to Ships of War but to have no Carlings let down at those Rooms to distinguish them, nor are there to be any Carlings let down between the Quarter Deck Beams, except the Rooms for the Mast and Ladderway, where half Beams may be necessary as the scores thereof are very hurtful to the Beams.  Those Carlings to be Oak, The Ladder Way from the Quarter Deck to be placed the Room Afore the Wheel, or as shall be directed.

LADDER WAYS	To make all such Ladder-ways as shall be directed.

SPIRKETTING	The Quarter Deck Spirketting to be 3 ½  inches English Oak Plank seasoned.

QUARTER DECK 	To have 7 Quarter Deck Ports on each Side, placed as on the Draught,

PORTS 	2 feet 8 inches fore and aft, and 2 feet 5 inches deep.

RING and EYE	To have 2 Ring and 2 Eye Bolts to each Port, the Bolts 1 inch

BOLTS 	diameter; the Rings 3¾ inches the clear.  To shut in between the Ports with 2 inches Prussia Deal.

STEERING WHEEL	To fix a proper double Wheel for steering the Ship, and a Breast Work at the Fore Part of the Quarter Deck agreeable to Ships of the Class, as fitted in His Majesty’s Yards.

ROUND HOUSE	The String or Round House Clamp to be 4 inches thick, beard to 3 inches

CLAMPS	at the lower edge and 13 inches broad, bolted with one under and one between each Beam with Bolts of ¾ inch diameter.

BEAMS	The Round House Beams to round 11 inches, to be sided 7 inches, and moulded 5 ½ inches placed as on the Draught tailed and bolted into the String with Bolts of ¾ inches diameter.

TRANSOM  	The Round House Transom to be 8 inches deep rabbited, and of a sufficient breadth to receive the ends of the Deck, scored & bolted to the Stern Tiber and kneed at each end with one Iron Knee bolted with 7 saucer headed Bolts of ¾ inch diameter, the fore and aft Arm of the Knee to be of a length sufficient to receive two Bolts before the foreside of the Gallery Door or as shall be directed.

KNEES	To knee every Beam with one hanging and one lodging Knee sided 5 inches of Wood at each end from the Bulkhead inclusive forward, the hanging Knees to reach the Spirketting, the other 2 feet 8 inches long bolted with 5 bolts & the lodging Knee with 4 Bolts of ¾ inch diameter and those beams from the Bulkhead of the Cabbin aft, to be Kneed with on hanging Knee of Iron & one lodging Knee of Wood, the Hanging Knee to reach the upper part of the Quarter Deck Ports.

WATERWAYS	The Waterways to be 4 inches Plank, and the Round house laid with 2 ½ inch Prussian Deals.

GRATINGS	To have a grating on the Round house 2 feet 9 inches in the Clear and a handsome Companion with Sash, to give light to the Coach.

TRANSOM for the	To have a Transom or Gunwale 5½ inches deep at the upper edge of the

ENSIGN STAFF 	Taffrail or Fife rail & proper Security for the Ensign Staff with proper Tumbler & Clasp.

FORECASTLE 	The Forecastle Clamps to be 4 inches thick, and of Breadth

CLAMPS 	sufficient to work down to the Ports, to scarph and lock with Hook and Butt to the String, and the Lower Edge to run fair with the Lower Edge of the String.

FORECASTLE 	The Forecastle Beams to round 6½ inches sided 9½ inches, and

BEAMS 	moulded 8 inches, placed as on the Draught, to be kneed at each End with One Hanging and One Lodging Knee, the Hanging Knee sided 6½ inches: The Up and Down Arm to reach  the Spirketting, the Thwartship’s Arm to be 3 feet 2 inches long; and bolted with7 bolts of ⅞ inch diameter  the Lodging Knees sided 6 inches, and both Hanging and Lodging Knee bolted with 6 Bolts of ⅞ inch diameter.

F’CASTLE PORTS	To have 2 Ports on the Forecastle on each side and one of each side right forward to be fore & aft and height from the Deck the same as those on the Quarter Deck.

WATERWAYS	The Waterways to be 4 inch Plank, and the Deck laid with 3 inches Prussia Deal.  To have one Strake of English Plank next the Waterways, the rest to be well seasoned Deal, not to work broader at the Aft Part than 10 inches.

SPIRKETTING	The Spirketting on the Forecastle to be 3½  inches Plank, and to leave Timber Heads proper for Shank Painters, Stoppers, and such Uses as they may be wanted for.

BEAM UNDER	The Beam under Cathead to be 2 feet 4 inches broad and 10 inches deep

the CATHEAD 	rabbetted abaft the Cathead for the plank of the Deck 3 inches deep & 5 inches broad, to be kneed with one lodging Knee and a Standard Knee under the Beam Side 11 inches

CATHEADS	The Catheads to be Fore and Aft 1 foot 5½ inches, up and down 1 foot 4½  inches, of sufficient length without Board and with 3 Shivers in each with Iron Pins & the shivers cogged the Cathead to be from the outside of the Timber in up the Beam not less the 8 feet 6 inches.  To have a piece of Oak wrought on the inner ends of them & both bolted to the Forecastle Beam with \Bolts of ¼ inch diameter and to be of a length from the side out 7 feet and to have a Knee on the aftside of 7 ¼ inches sided, fore & aft Arm 4 feet 9 inches long thwartships 3 feet, bolted with Bolts of ⅞ inch diameter.

BEAKHEAD	The Stantions of the Beakhead to be 6½ inches square, well stopped with a Double Stop in the Collar Beam, scored & shouldered half an inch on the Forecastle Beams and bolted with 2 Bolts in each with Bolts of ¾ inch Deal, or to run down on the Deck as before observed.

FURNACE and FIRE	To lay Cants for the Iron Fire-Hearth; to line the Deck and

PLACE	 Beams overhead round the said Fire hearth and the foreside with double Tin Plates, and the Forecastle Beams with Lead and Tin, and to cover the Flat of the upper deck with Lead 7 lb. to the foot square & to put up such convenience, Dresser, & Lockers, and do everything that shall be required for the conveniency & safety of the Ship from Fire & use of the Forecastle for cooking.

FORE JEER & TOP-    The Fore Jeer & Top Sail Sheet Bitts to be 12 inches square and 3 feet 8

SAIL SHEET BITTS	inches above the Deck, the Cross Piece 7 inches deep by 9 inches broad, score 1 ½ inches on the Bitt pins.

SPANSHACKLE	The Spanshackle Bolts to be 1⅛ inch in the Forecastle Beam, & 1⅝ inches in the upper Deck Beams, the shackle of sufficient bigness for the Forecastle through which the Spanshackle is drove to be left 2 inches broad in the middle to answer its being cut by the Bolts.

EYE BOLTS	The Eye Bolts for the Main Topmast stay of 1¼ inch diameter those on the Deck & about the Mast by 1⅛ inch diameter.

BULKHEAD 	To lay Cants & build the Bulkheads of the Forecastle & Roundhouse with proper Lights & Doors complete.

GREAT CABBIN	To part off and finish with Joiner’s Work, a great Cabbin, Bed Place, and Lobby under the Quarter Deck, the Bulkheads to be hung with hinges, & to be Cieled with Slit Deal between the Beams overhead both in the Great Cabbin, Steerage, & Stateroom the Ship’s side & Decks in the Great Cabbin, Bedplace, & Lobby in the Roundhouse to be wrought with clean stuff fayed close, Caulked & plained and the Ship’s Side puttied & painted.  To fix proper stiles of 4 inch plank rabbited in the Bulkheads to prevent their working at Sea, & to finish the said Cabbin & Bedplace equal to the Ships of her Class in the King’s Yard & as shall be directed by the proper Officers appointed to inspect the same, & to have Wainscott Doors to the Cabbin & those in the Galleries.  To have proper Dead Lights to the Stern & Gallery Doors, with sufficient Bolts &c. for securing them.

OFFICERS CABINS	All the Officers’ Cabbins to be put up in grooves that they may with little trouble be taken down & put up again as is done in His Majesty[‘s Yard, & to finish the insides with Bedplaces, lockers, & sashes as is usually fitted.

BEAMS to be	All the Beams & Knees of the Upper Deck, Quarter deck, Forecastle, &

PLAINED	Roundhouse to be plained & struck with a Bead, the Ship’s sides within board & undersides of the Deals of the Upper Deck, Forecastle & Quarterdeck to be plained & Tin Shakes all puttied & filled up before painted.

LADDERS and 	To make all Ladders, Gratings, Hatches, Gangways, Iron Staircases,

GRATINGS 	and to fix all necessary Rolls, Stantions, Blocks, Ranges, and Kevels, as shall be necessary, and directed by the Officers or Overseers appointed to inspect the same.  To build all such Cabbin, Pantries, & Conveniences, & furnish the same with all needful Locks, Hinges, Latches, Scuttle Lights, with stone ground Glass as shall be directed.

ROTHER	The Rother-head to be made long enough to receive a Tiller above the Upper Deck, to be strapped and hooped with Iron as usual, to be 2 feet 2 inches Thwartships: Fore and Aft 2 feet 4 inches, and 5 feet 6 inches broad at the lower End, and 4 feet 6 inches at the lower Hance height above the Deck as the Draught shall Direct. To be well made, the Pieces tabled to each other, and all but the Main and Bearding Pieces, to be Fir.   To have 7 Pair of Rother Irons, the upper brace to have long Straps, that may turn and meet round the Post.  The second Brace 4 feet 6 inches long from the Rabbit, the lower one from the Back of the Post 7 feet; to be hung Flemish Fashion, and well secured with Chocks above Water to prevent its unhanging.                                                                                                     	The Pintles to be 3⅜ inches diameter, all of them to be 1 foot 1½   inches long, except the lower one, that to be 1 foot 3½  inches long; the Straps of the Rother Braces and Pintles to be 4½  inches broad, and 2 inches thick in the Shoulder of the Return, and to have an iron strap on the Back of the Rother, and a Ring Bolt with two Rings by 1⅞ inches diameter drove through the Rother; the Rings of sufficient bigness; and to have an Eye bolt on each Side at the Ends of the Wing Transom, with Bolts of 1¼  inches diameter for the Rother Tackles, that all, both Braces and Pintles be carefully let in, that the Irons bear an equal Strain, that every Pintle and Brace have Bolts in the strap as close to the Shoulder as may be, drove through with a Saucer-head, and well clenched on the other side, the hole for the Tiller in the Rotherhead to be cut both that above & below the Upper Deck of equal bigness, square on the foreside 11⅛ inch on the aft side 10 inches that the same Tiller may fit either hole.  The Bearding of the Rother, between the Pintles and Back of the Stern Post, to be covered with Lead turned and nailed on the Sides; also to Lead the Helm Port, as is done in His Majesty’s Yards.

MAIN CHANNELS	The Main Channels and Back Stay Stools to be 33 feet long, placed above or below the Upper Deck Ports, as shall be directed, of sufficient breadth to carry the Shroud 18 inches clear of the Fife rails; the inner Edge 5¾ inches, the outer Edge 4½ inches thick, bolted with 8 Bolts of 1¼  inches diameter.

FORE CHANNELS	The Fore Channels and Back Stay Stools to be 31 feet long, placed above or below the Upper Deck Ports, as shall be directed, of sufficient breadth to carry the Shrouds 1 foot 8 inches clear of the Gunwales, the inner Edge 5¾ inches the outer Edge  4½ inches thick, bolted with 8 Bolts of 1¼  inches diameter.

MIZEN CHANNEL	the Mizen Channels to be 18 feet long, of sufficient breadth to carry the Shrouds 15 inches clear of the Fiferails,  the inner Edge 4½ inches, the outer Edge 3½ inches thick, bolted with 5 Bolts of 1 inch diameter

KNEES in 	To have 6 Knees to the Main and Fore Channels, sided    4 ½  inches

the CHANNELS 	inches, bolted  with 4 Bolts in each with Bolts of  ¾  inches diameter, with proper Knees or Spurs in the Mizen Channels.

CHAIN PLATES	To have Chains, instead of Chain Plates, as shown on the Draught of sufficient Strength, for the Chain Bolts to come through the Upper Deck Spirketting, as low down as can with Ease be drove out, and the Preventer Bolts to either in the Upper Deck Clamp, or short Stuff between the Ports.

The size of the Iron for the Links to be 1⅝ diameter or as shall be directed, and made out of the best Sort of Orgrounds Iron, wrought with all imaginable care; the Chain Bolts 2⅛ inches diameter, and Preventer bolts 1⅝ inches diameter.

MIZEN CHAIN & 	The Mizen Chain and backstays to be also Chains as shall be directed.

BACK STAY PLATES	 The Iron of the Links to be 1⅜ inch diameter, the chain Bolts 3 feet  below the Channel, and 1⅝ inches diameter: To have two Ring Plates to each Channel, with Swivels, Eyes, and Hooks for Studding Sail Booms, and all Iron-work both in Main, Fore, and Mizen Channels, that is required for a Ship of the Class, and shall be directed to be provided.

BINDINGS	The Bindings of the Dead-Eyes in the Main and Fore Channels to be 1⅝ inches diameter, those for the Mizen Channels, and for the Backstays and Ring Plates, in the Main and Fore Channels to be

1¼  inches diameter, made out of the best Sort of Orgrounds Iron, wrought with the greatest Care, that no Flaw, Burn, or defect of any Kind appear therein.

DEAD EYES	To have on each Side 11 Dead-Eyes of 16 inches diameter in the Main Channels, 11Dead-Eyes of 16 inches diameter in the Fore Channels, their Thickness One Inch more than Half their Diameter. To have on each Side 6 Dead Eyes of 10 inches diameter in the Mizen Channels,  3 Deadeyes of 10 inches for the Main Topmast Back stays, 3 Deadeyes of 10 inches for the Fore Topmast Back stays & 3 Dead Eyes of 7 inches for the Mizen Topmast, Main & Fore Top Gallant Back stays their Thickness half an inch more than Half their Diameter.  The Dead-Eyes to be made out of the best Sort of Elm, sound and good, to be wrought full in the Quarters, and the Holes well disposed and suitable to the Lanyards.

SHANK PAINTER	The Shank Painter Chains to be 17 feet 6 inches long.  The Links 15/16 inches diameter.  The Gun Deck Port Hinges 3 ¾ inches broad & 1 inch thick, the Port Hooks to be made from a Mould that shall be given of 1 ¼ inch diameter made out of the best sort of Iron and clenched within side.

PORT SHACKLES 	 To have 2 Port Shackles to each Port well clenched on the hinges of 1 inch diameter. The Rings 2/3/4 inches in the clear, to have 2 Shackles to each Port for the Port Ropes to go through the Hinges, & clenched on the inside of the port ⅞ inch diameter the Rings 2¾ inches in the clear.

EYE BOLTS	The Eye-Bolts for the standing Part of the Main and Fore Sheets of 1⅝ inches diameter; for the Mizen Sheet main brace, Topsail Hallyards, and Bowsprit Shrouds of 1⅜ inches diameter; for the Mizen Topsail Hallyards, and Mizen Truss, Burton and Stern ladders of 1¼ inches diameter.

SWIVEL for LONG	The Swivel for the Long-boat of 1⅜ inches diameter.  The Rings

BOAT 	4½ inches in the clear.

BOOM IRONS	To have Studdingsail Boom Irons fixed in the Main and Fore Channels, and an Eye-Bolt forward in the Side, almost before the Fore Channels, for the Studdingsail Boom, with Ring Plates, Swivels, and all iron Work usually fitted in the Channels; also a hoop and swivel for the Driver Boom abaft on the Quarter.

CHESS TREES	The Chess-trees to be sided 7 inches, and bolted with 4 bolts of ⅞ inches diameter; the Hole for the Tack to be below the Sheer Rail, or so as to be clear of the Gun, and a Block let through the Side, agreeable in Height, for the Tack to come through, and answerable to the hole in the Chess-tree.

FENDERS	The Fenders to be placed right against the Hatchways, sided 4  inches,  and      1 foot 4 inches asunder.

RAILS	The Sheer Rail to be 8 inches broad, and 3 ½ inches thick; the Waste Rail 7       inches broad, and 2¾ inches thick; the Rails in the Drift to be 6 inches broad and 2½ inches thick.

FIFERAILS	The Fiferails to be Oak 11 inches broad and 3 ½ inches thick, cyphered down to 3 inches at the Edges.

LININGS of the 	The linings of the Anchors to be 3 inches Plank, of Length and Form as

ANCHORS	 shall be directed.

BOLSTER	To have a bolster on each side instead of Linings to the Hawse to work up to the middle of the holes, to be in thickness so as to project ¾ inch over the Cheeks. To be fastened with Bolts of an inch diameter.  XXXXXXXXXXXX

KNEE of the HEAD	The knee of the head to extend to the upper Side of the upper Cheeks, to be 1 foot 5 inches inches thick at the stem; the two upper Bolts to be 2⅜ inches diameter in the Knee, and 2 ¼ inches in the Stem; the third 2⅛ inches diameter in the Knee, and 2 inches in the Stem; the fourth to be 1⅞ inches in the Knee, and 1¾  inches in the stem; the fifth to be 1 ¾ in the Knee & 1⅝ in the stem. and all the rest to be 1½ inches diameter.  All the Holes to be bored through with proper size Augers, that is, to receive the Bolts in the least Drift, that no Reeming may be suffered from the Stem inwards; that the Holes be bored before the Canvas is put on, and the Knee swung off, to observe if the Holes are all good, before any Bolts are drove: To have a Throat Piece of sufficient Depth of Elm, tabled and billed, and well secured with Bolts drove up with

GRIPE	To lead the Gripe and front with thick Lead of 12 Pounds to the Foot square, to lap on each Side 6 inches, and buried in a Rabbit fastened with proper nails, their Head dipped in Lead, and to run 8 feet 0 inches abaft the End of the Keel, so high as the lower Cheek the ends to be let into the Wood, and well fastened: To have a Horseshoe to the Gripe properly fastened.

CHEEKS	To have Two Cheeks to the Knee of the Head as usual; the lower Cheek sided 12 inches, the upper 11 inches, the Arms next the Side 11 feet 9 inches long, bolted with Bolts the same sized Bolts as the Gun Deck Knees.

STANDARD in the 	To have a well grown Standard in the Head, sided 1 inches, and

HEAD	 sufficiently fastened.

RAILS of the HEAD	The upper rails of the Head, at the after End, to be sided 9½ inches, and moulded 12 inches; the Timbers in the Head to be sided from 7½ inches to 5 inches. To have a thin Lining on the upper Rail, with a false Rail as usual.

TRAILBOARD 	To have a double Trail Board and a Lyon or Figure, as shall be directed,

Bust	handsomely carved; the Rails and Supporters handsomely wrought with Mouldings.

IRON HORSE in 	To have an Iron Horse in the Head on each side supported in the Middle with one or two Ion Stantions as required.  To fix Bumpkins for the Fore tracks; Cut holes for the Gammoning & to finish the head with Timbers, Grating, Seat and Fenders, all complete in the handsomest manner & equal to Ships of her Class, built in His Majesty’s Yards.

GALLERIES	To build and complete a handsome Light Gallery on each Side, with convenient Places of Easement, Lights, and Lead Gunnels, according to the draught or as shall be directed by the  Officers appointed to inspect the same.  To lead the seams under the Galleries & the aft part of the Taffrail as shall be directed and that each of the Upper Stools be caulked and covered with Canvas, and upon that Sheet Lead 5 Pounds to the Foot Square. To have Lead Pipes in the Quarter Galleries, from the upper part of the upper stool to the underside of the Lower Finishings.

COUNTER		To have 6 Counter or Stern Timbers, sided 10 inches properly placed to

TIMBERS 		make the Stern Ports & Lights to run down 6 inches below the upper edge 				of the Wing Transom scored and tailed half their thickness into the Wing 				the Wing Transom & to step one inch down on the upper side bolted with 				1 bolt of 1 inch diameter with such other short timbers to strengthen the 				counter as shall be necessary.  To birth up the lower Counter with 3 inch 				plank the Rabbet cyphered off and the upper Counter with 2 inches Oak 				Planks rabbetted.

STERN	As the Stern is to be open with Walk to have proper Knees fixed on the Roundhouse for the Ensign Staff and security of Taffrail 4 in number to be sided on the Deck 6 ½ inches &at the upper end 5 ½ inches.  The fore and aft Arm to reach the third Beam & properly fastened.

TAFFRAILS and 	To have a handsome Taffrail and Quarter Pieces neatly carved, to birth up

QUARTER PIECES	the Stern under the Taffrail, with 3 inches Oak plank rabetted and put in flush and bolted to the side Timber.  The Stern Rails to be wrought with proper Mouldings, the quarter Pieces to have proper Squared Irons and the Stern completed with sash Lights and Stone Ground Glass, to slide, with handsome Walk or balcony with Ballisters or as shall be directed buy the Draught, and the whole Stern finished agreeable to Ships of her Class built in His Majesty’s Yards.  The taffrail and head of the Quarter Pieces as also the inside of them to be loaded where necessary to keep out the Water.

GLASS	All the Stone Ground Glass for the Stern Galleries, Bulkheads, and Scuttles, to be 1/6 inch thick, clear and well polished, and the raking Panes nowhere to exceed 16 inches in Length measure on the Square.

PANTRIES	To build all such Pantries and Conveniences, and furnish the same with all needful Locks, Hinges, Latches, Scuttle Lights, with Stone Ground Glass, as shall be directed and all other Locks in the Ship.

TACK PIECES &c	To fix Bumpkins for the Fore Tack, cut Holes for the Gammoning, and to finish the Head with Timbers, Gratings, Seats and Fenders, all compleat, in the handsomest Manner, and equal to Ships of her Class built in His Majesty’s Yards.

Not to FIND		And it is hereby Agreed, that we are not to the Masts and Yards

MASTS &c.		nor provide, nor set the Furnaces or pumps, & Pump Dales or   Iron Fire 				Hearth.

TIMBER AND	That all Timber and Plank shall be of the Growth of England, except the

PLANK	Plank of the Bottom under the light Draught of water, which may be East Country Plank, provided it be good white Crown Plank, otherwise to be English Plank; all the Crutches,  Knees, Standards, Riders, Crutches and Breast hooks to have full a faying of Spine against the respective Beams, Sides, Decks, Ceilings, &c. that the Bolts for fastening of each may not be naked when the Sap rots away, as has too frequently appeared to the evident weakening of Ships formerly built in Merchants Yard, and has proved the Principal Means of their too soon Decay: and also the whole Frame to be well grown, square edged, free from Sap, Shakes, and Defects; and that all the Plank of the Bottom, Sides, and Decks shall be good yellow wood free from Sap, Shakes, and Defects, and that no Sap shall be wrought in the Edges of them.

IRON WORK	All Iron Work shall be wrought out of the best tough Swedish Iron, not burnt or hurt in working; all the Bolts to be clenched, or belayed, as shall be directed.  Those to the Iron Knees and Standards to be drove through them into the Transoms, Beams, or Timbers, and all clenched on Rings let into the wood.

TREENAILS	All  the Treenails to be dry seasoned, clear of Sap, and converted from Timber of the growth of Sussex, or equal in goodness, therefore, to be well mooted, not overhauled with an Axe in driving, and all to be caulked or wedged at both Ends.

CAULKING	The Ship’s sides and Decks to be caulked: To drive in every Seam and butt of Eight Inches Twelve double Threads of Ocham, and Two of Hair; in every Seam and butt of Seven Inches ten double Threads of Ocham, and two of Hair; ; in every Seam and butt of Six Inches eight double Threads Ocham, and two of Hair; in every Seam and butt of Five Inches Six double Threads of Ocham, and two of Hair; in every Seam and butt of Four Inches Five double Threads of Ocham, and two of Hair; in every Seam and butt of Three  Inches Four double Threads of Ocham, and One of Hair;  in every Seam and butt of Two Inches Two double Threads of Ocham, only. The first double thread of Ocham may be white, and to have no more than one double Thread of white Ocham in any Seam, the white Ocham to be from Filings, not from Ground Toes or decayed white Ropes, the black Ocham to be picked out of good Junk & both to be good sound Ocham. All the Work in wake of the Cabbins, Bulkheads, Cants, Standards, Chock of the Hawses & Lining of the Anchors, to have a second Caulking before they are finished for good.

GRAVING	To Grave the Ship with the usual Graving of White Stuff under Water taking care to Bream & Dry the Bottom very well before it’s laid on & to pay the sides from the Water upward with tallow or  Varnish as shall be directed.

PAINTING	To treble paint the Ship without Board with good Oil Colours, and in like Manner the Great Cabin, Steerage, and all the Cabins, Rails, Stantions, Ladders, Gratings, Gangways, Bulkheads, Capstands, Upper deck beams, and Knees, and all the Work both within and without board for finishing and compleating the Hull of said Ship, shall be done in a Workmanlike Manner, equal in all respects to those of His Majesty’s Ships of the like Quality in the King’s Yards.

DEFECTS to be	And that right may be done to his Majesty in every part of the said

AMENDED 	Workmanship, we do oblige ourselves in case the Officer appointed by the principal Officers & Commissioner to inspect the same, shall at an time find an defection or unsound Materials, or insufficient Workmanship perform to the said Ship during her building, the said Deficiency both in the one and the other shall upon notice thereof be forthwith amended & the said Officer or Officers shall not at any time have any molestation of obstruction thereon.

Time of LAUNCHING  And we do oblige ourselves that the Frames, Beams, Knees and thickstuff shall be prepared & completed by or before twenty six Months from the date hereof that the Frames shall stand to Season Sox Months if not otherwise ordered, & that all the Works of the said Ship shall be performed& perfected as before mentioned agreeable to the Draught for Draughts that accompany this Contract Launched and putsafe into the Hands of such Officer or Officers as shall be appointed to receive her, by or before the expiration of Forty Four Calendar Months from the Date hereof, but in case the said Commission should direct that the said Ship should not attend to season we do oblige ourselves to complete and launch her by or before the expiration of Thirty Eight Calendar months from the date hereof.

RATE per TON	For which we are to be paid   after the Rate of 17.10.0 Pounds for each Ton, namely, for so many Tons as the said Ship shall measure, not

Exceeding one thousand six hundred and thirty four Tons & 21/94 part of a Ton but in the case the discount on Navy Bills shall be above two and a half per cent at the time any Bills shall be made out to us in pursuance of the Contract, an advance shall be added to each Bill of so much money as the discount above the said two & a half percent on such Bill shall amount to, the rate of which discount to be ascertained by <Castaings> paper the price of Stocks last published preceding the day of the date of the Bill.  It is also agreed that no allowance is to be made in case the said Ship shall exceed in dimensions & Scantling contrary to what is here in set forth and declared unless such increase of Scantlings and dimensions shall be made by order first given in Writing by the said Principal Officers and Commissioners of His Majesty’s Navy, and the before recited conditions of payment are hereby allowed and accepted between the parties in this Contract,  the full total and entire compensation and payment for the said Ship, without any other Demand, Charge, or Expense whatsoever, whatsoever.